LibUHFReader.so Dynamic Link Library User ManualV1.5

catalogue

catalogue	2

1. Introduction	4

2. Application integration	5

3. Programming interface	6

3.1 Connect/Close Reader/Writer	6

3.1.1 AutoOpenComPort	6

3.1.2 OpenComPort	7

3.1.3 CloseComPort	8

3.1.4 CloseSpecComPort	9

3.1.5 OpenNetPort	10

3.1.6 CloseNetPort	11

3.2.18000-6C standard command	12

3.2.1.Inventory_G2	12

3.2.2.InventoryMix_G2	14

3.2.3.InitRFIDCallBack	16

3.2.4.ReadData_G2	17

3.2.5.WriteData_G2	18

3.2.6.BlockWrite_G2	19

3.2.7.ExtReadData_G2	20

3.2.8.ExtWriteData_G2	21

3.2.9.WriteEPC_G2	22

3.2.10.BlockErase_G2	23

3.2.11.Lock_C2	24

3.2.12.KillTag_G2	26

3.2.13.SingleTagInventory_G2	27

3.2.14.StartRead	28

3.2.15.StopRead	29

3.2.16.Authenticate_G2	30

3.3.Reader/writer custom commands	31

3.3.1.GetReaderInformation	31

3.7.2 SetAddress	32

3.7.3 SetInventoryScanTime	33

3.7.4.SetRfPower	34

3.7.5.SetRegion	35

3.7.6.SetBaudRate	36

3.7.7.BuzzerAndLEDControl	37

3.7.8.SetAntennaMultiplexing	38

3.7.9.SetAntenna	39

3.7.10.SetBeepNotification	41

3.7.11.SetReal_timeClock	42

3.7.12.GetTime	43

3.7.13.SetRelay	44

3.7.14.GetGPIOStatus	45

3.7.15.SetGPIO	46

3.7.16.SetNotificationPulseOutput	47

3.7.17.GetSeriaNo	48

3.7.18.SetCheckAnt	49

3.7.19.GetTagBufferInfo	50

3.7.20.GetModuleVersion	51

3.7.21.SetFlashRom	52

3.7.22.WriteRfPower	53

3.7.23.ReadRfPower	54

3.7.24.RetryTimes	55

3.7.25.SetReadMode	56

3.7.26.SetDRM	57

3.7.27.GetDRM	58

3.7.28.GetReaderTemperature	59

3.7.29.MeasureReturnLoss	60

3.7.30.SetRfPower	60

3.7.31.GetRfPower	61

3.7.32.SetCfgParameter	62

3.7.33.GetCfgParameter	63

3.7.34.SetProfile	64

3.7.35.StopImmediately	65

3.7.36.SetExtProfile	66

3.7.37.GetEx10Version	68

4. Reader/writer parameter configuration	69

4.1 Controller Configuration Parameter Table	69

4.2 Reader and writer parameter function description	70

4.2.1 CFG7: Inquiry Parameters	70

4.2.2 CFG: TagFocus switch	70

4.2.3 CFG9：Q and Session	70

4.2.4 CFG 10: TID Inquiry Parameters	71

4.2.5 CFG 11: Mask Parameters	71

4.2.6 CFG25：Impinj Scan Parameters	71

4.2.7 CFG29：Impinj ScanID Parameters	72

4.2.8 CFG 31: Scene Mode Custom Profile	72

Appendix 1. Return Value Error Code Table	73

Appendix 2. Error Code Table	74

- 1. Introduction

This development package supports C, C++, C #, and other languages that can call C function interfaces.This development package currently only supports the Linux operating system.

The upper computer application can operate R2000 series and domestic C6 series readers and writers through libUHFReader.so, supporting the upper computer application to simultaneously open multiple readers and writers on multiple communication ports. This manual describes the product's API interface, which is not supported for all products.

2. Application integration

This development package consists of the following files:

| file | use |
| --- | --- |
| libUHFReader.so | SO dynamic link library, including all API interfaces |
| UHFReader.h | C project header file, function definition |

| programming language | describe |
| --- | --- |
| VC++ | Copy libUHFReader.so to the Linux executable output directory. |
| C# | Copy RWDev. cs to the project directory and add it to the project.Copy libUHFReader.so to the Linux executable output directory. |

3. Programming interface

3.1 Connect/Close Reader/Writer

### 3.1.1 AutoOpenComPort

| definition | int AutoOpenComPort(char* port, unsigned char* ComAdr, unsigned char baud, int* FrmHandle); |  |  |
| --- | --- | --- | --- |
| describe | Automatically recognize and open the serial port connected to the reader/writer. This feature only supports serial ports between/dev/tty0-/dev/tty9,/ttyS0-/dev/ttyS9, and/ttyUSB0-/dev/ttyUSB9. |  |  |
| parameter | name | type | notes |
|  | port | char* | Return the serial port number connected to the reader/writer. |
|  | ComAdr | unsigned char* | Enter the reader/writer address. If the real address is unclear, you can enter the broadcast address 0xFF. After the function call is successful, this parameter will return the real address. |
|  | baud | unsigned char | Serial communication baud rate.0-9600；1-19200；2-38400；5-57600；6-115200；7-921600. |
|  | FrmHandle | Int* | Return the handle connected to the reader/writer, which is required for all API operations thereafter. |
| return (int) | Success: 0;Failure: non-zero;(View the table of error codes for return values) |  |  |
| Reference code | unsigned char port[16]={}; unsigned char address=255;//Broadcast address unsigned char baud=1;//19200 int FrmHandle=-1; int fCmdRet =AutoOpenComPort(port,&address, baud, &FrmHandle); |  |  |

### 3.1.2 OpenComPort

| definition | int OpenComPort(Char*port, unsigned char* ComAdr, unsigned char baud, int* FrmHandle) ; |  |  |
| --- | --- | --- | --- |
| describe | Open the serial port connected to the reader/writer. |  |  |
| parameter | name | type | notes |
|  | port | Char* | The serial port number connected to the reader/writer./dev/tty*、/dev/ttyS*、/dev/ttyUSB*… |
|  | ComAdr | unsigned char* | Enter the reader/writer address. If the real address is unclear, you can enter the broadcast address 0xFF. After the function call is successful, this parameter will return the real address. |
|  | baud | unsigned char | Serial communication baud rate.0-9600；1-19200；2-38400；5-57600；6-115200；7-921600. |
|  | FrmHandle | Int* | Return the handle connected to the reader/writer, which is required for all API operations thereafter. |
| return (int) | Success: 0;Failure: non-zero;(View the table of error codes for return values) |  |  |
| Reference code | Char port[]=”/dev/ttyS0”;//Open the serial port/dev/ttyS0 unsigned char address=255;//Broadcast address unsigned char baud=5;//57600 int FrmHandle=-1; int fCmdRet =OpenComPort(port,&address, baud, &FrmHandle); if(fCmdRet==0) { //Connection successful ........... } |  |  |

### 3.1.3 CloseComPort

| definition | int CloseComPort(); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | Revoke all serial port and reader connections and release the corresponding resources. |  |  |  |
| parameter (None) | name | type | direction | notes |
| return (int) | Success: 0;Failure: non-zero;(View the table of error codes for return values) |  |  |  |
| Reference code |  |  |  |  |

### 3.1.4 CloseSpecComPort

| definition | int CloseSpecComPort(int FrmHandle); |  |  |
| --- | --- | --- | --- |
| describe | Revoke the connection between the specified serial port and reader/writer and release the corresponding resources. |  |  |
| parameter | name | type | notes |
|  | FrmHandle | int | Handle connected to the reader/writer, handle obtained by opening the serial API. |
| return (int) | Success: 0;Failure: non-zero;(View the table of error codes for return values) |  |  |
| Reference code |  |  |  |

### 3.1.5 OpenNetPort

| definition | Int OpenNetPort(int Port,char* IPaddr, unsigned char*ComAdr, int *Frmhandle); |  |  |
| --- | --- | --- | --- |
| describe | Open a TCP connection with the reader/writer. |  |  |
| parameter | name | type | notes |
|  | port | int | The port number connected to the reader/writer. |
|  | IPaddr | char* | The IP address of the reader/writer. |
|  | ComAdr | unsigned char* | Enter the reader/writer address. For TCP connections, you can directly enter the broadcast address 255. |
|  | FrmHandle | Int* | Return the handle connected to the reader/writer, which is required for all API operations thereafter. |
| return (int) | Success: 0;Failure: non-zero;(View the table of error codes for return values) |  |  |
| Reference code | int port=6000; char ipAddr[] =”192.168.1.190”; unsigned char ComAdr =255;//Broadcast address int FrmHandle=-1; int fCmdRet =OpenNetPort(port, ipAddr ,&ComAdr, &FrmHandle); if(fCmdRet==0) { //Connection successful .......... } |  |  |

### 3.1.6 CloseNetPort

| definition | int CloseNetPort(int FrmHandle); |  |  |
| --- | --- | --- | --- |
| describe | Close the TCP connection with the reader and release the corresponding resources. |  |  |
| parameter | name | type | notes |
|  | FrmHandle | int | Handle connected to the reader/writer, handle obtained by opening a TCP connection. |
| return (int) | Success: 0;Failure: non-zero;(View the table of error codes for return values) |  |  |
| Reference code |  |  |  |

3.2.18000-6C standard command

### 3.2.1.Inventory_G2

| definition | int Inventory_G2(unsigned char * ComAddr,unsigned char QValue,unsigned char Session,unsigned char MaskMem,unsigned char *MaskAdr,unsigned char MaskLen,unsigned char *MaskData,unsigned char MaskFlag,unsigned char AdrTID,unsigned char LenTID,unsigned char TIDFlag,unsigned char Target,unsigned char InAnt,unsigned char Scantime,unsigned char FastFlag,unsigned char *pEPCList,unsigned char *Ant,int *Totallen,int *CardNum,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | Check if there are electronic tags within the valid range that comply with the protocol. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | QValue | unsigned char | [in] | 1 byte. Bit7: Statistics packet flag. 0- After the inquiry is completed, do not send the inquiry process statistical data packet; After the inquiry is completed, send a statistical data packet for the inquiry process. Bit5: Quick ID query flag, only supports Impinj Monza series partial tags, used to quickly obtain EPC and TID of tags.0- Not enabled;1- Enable. Bit4: Return phase information flag.0- Do not return;1- Return. Bit3~bit0: The initial Q value used when querying EPC tags. Note: 1. The Q value should be set to the power of Q where the number of tags in the field is approximately equal to 2.The range of Q value is 0-15. If other values appear in the command, a message indicating parameter error will be returned. 2. The return query speed with statistical package is slower than without it. |
|  | Session | unsigned char | [in] | 1 byte, the Session value used when querying EPC tags. 0x00: Session uses S0; 0x01: Session uses S1; 0x02: Session uses S2; 0x03: Session uses S3. 0xff: Reader/writer automatically configures session (only valid for EPC queries) Suggest selecting S0 for single or small tag queries. |
|  | MaskMem | unsigned char | [in] | 1 byte, masked area. 1- EPC storage area; 2- TID storage area; 3- User storage area. |
|  | MaskAdr | unsigned char* | [in] | 2 bytes, the starting bit address of the mask (in bits).Range 0~16383. |
|  | MaskLen | unsigned char | [in] | 1 byte, the bit length of the mask (in bits). |
|  | MaskData | unsigned char* | [in] | N bytes, masked data.N is equal to MaskLen/8.If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded to the nearest integer plus 1.If it's not enough, add 0 at the low level. |
|  | MaskFlag | unsigned char | [in] | 1 byte, mask flag bit.0- No mask;1- Mask. |
|  | AdrTID | unsigned char | [in] | 1 byte, query the starting word address of the TID area. |
|  | LenTID | unsigned char | [in] | 1 byte, query the number of characters in the TID area, ranging from 0 to 15. |
|  | TIDFlag | unsigned char | [in] | 1 byte, TID query flag bit. 0- Read EPC; 1- Read TID. |
|  | Target | unsigned char | [in] | 1 byte, query the Target value of the tag. 0 – Target A； 1 – Target B。 |
|  | InAnt | unsigned char | [in] | 1 byte, the antenna number to be queried this time. 0x80- Antenna 1;0x81- Antenna 2; 0x82- Antenna 3;0x83- Antenna 4; 0x84- Antenna 5;0x85- Antenna 6; 0x86- Antenna 7;0x87- Antenna 8; 0x88- Antenna 9;0x89- Antenna 10; 0x8A - Antenna 11;0x8B - Antenna 12; Single port reader/writer fixed at 0x80. |
|  | Scantime | unsigned char | [in] | 1 byte, the maximum time for this command query, range 0-255 (* 100ms). When the time is 0, it means there is no limit. |
|  | FastFlag | unsigned char | [in] | 1 byte, a flag for quick query. 0- Not enabled, no need to specify Target, Inant, Scantime has 3 parameters, default is 0; 1- Enable, at this time, you need to specify three parameters: Target, Inant, and Scantime. |
|  | pEPCList | unsigned char* | [out] | The returned label data is in the following format: EPC/TID length+EPC/TID number+RSSI.When there are multiple labels, they are returned by arranging several data in the same format in order. If the length of bit6 is 1, add 7 bytes of phase frequency information after the RSSI of the tag, starting phase (2 bytes)+ending phase (2 bytes)+frequency (3 bytes) |
|  | Ant | unsigned char* | [out] | 1 byte, query the antenna of the tag. 4-port reader/writer, each bit represents the antenna number, such as 0x04, binary is 0000 0100, indicating that antenna 3 queried the tag. 12 port reader/writer, corresponding to antenna 1 to antenna 12 according to the numbers 0-11. |
|  | Totallen | int* | [out] | The total length of data received by pEPLSist. |
|  | CardNum | int* | [out] | The number of tags found in this query. Note: When using callback method to return, Totallen and CardNum are both 0. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0x01: Command execution ends and returns the queried electronic tag data; 0x02: Inquiry time ends, command execution forces exit, and returns the queried label data 0xF8: Antenna connection detection error, the current antenna connection may have been disconnected; For the remaining values, please refer to the table of error codes returned. |  |  |  |
| Reference code | None. |  |  |  |

### 3.2.2.InventoryMix_G2

| definition | int InventoryMix_G2(unsigned char * ComAddr,unsigned char QValue,unsigned char Session,unsigned char MaskMem,unsigned char *MaskAdr,unsigned char MaskLen,unsigned char *MaskData,unsigned char MaskFlag,unsigned char ReadMem,unsigned char* ReadAdr,unsigned char ReadLen,unsigned char* Psd,unsigned char Target,unsigned char InAnt,unsigned char Scantime,unsigned char FastFlag,unsigned char *pEPCList,unsigned char *Ant,int *Totallen,int *CardNum,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | Check if there are electronic tags within the valid range that comply with the protocol, and read the specified tag data while querying the tag EPC number. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | QValue | unsigned char | [in] | 1 byte. Bit7: Statistics packet flag. 0- After the inquiry is completed, do not send the inquiry process statistical data packet; After the inquiry is completed, send a statistical data packet for the inquiry process. Bit4: Return phase information flag.0- Do not return;1- Return. Bit3~bit0: The initial Q value used when querying EPC tags. Note: 1. The Q value should be set to the power of Q where the number of tags in the field is approximately equal to 2.The range of Q value is 0-15. If other values appear in the command, a message indicating parameter error will be returned. 2. Sending packets with statistics is slower than without. |
|  | Session | unsigned char | [in] | 1 byte, the Session value used when querying tags. 0x00: Session uses S0; 0x01: Session uses S1; 0x02: Session uses S2; 0x03: Session uses S3. Suggest selecting S0 for single or small tag queries. |
|  | MaskMem | unsigned char | [in] | 1 byte, masked area. 1- EPC storage area; 2- TID storage area; 3- User storage area. |
|  | MaskAdr | unsigned char* | [in] | 2 bytes, the starting bit address of the mask (in bits).Range 0~16383. |
|  | MaskLen | unsigned char | [in] | 1 byte, the bit length of the mask (in bits). |
|  | MaskData | unsigned char* | [in] | N bytes, masked data.N is equal to MaskLen/8.If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded to the nearest integer plus 1.If it's not enough, add 0 at the low level. |
|  | MaskFlag | unsigned char | [in] | 1 byte, mask flag bit.0- No mask;1- Mask. |
|  | ReadMem | unsigned char | [in] | 1 byte, select the storage area to be read, 0x00- Reserved area; 0x01- EPC storage area; 0x02- TID storage area; 0x03- User Storage Area.Other values are retained.If other values appear in the command, a message indicating parameter error will be returned. |
|  | ReadAdr | unsigned char* | [in] | 2 bytes, to read the starting word address of the stored data, the high byte comes first. |
|  | ReadLen | unsigned char | [in] | 1 byte, the length of the word to read the stored data. |
|  | Psd | unsigned char* | [in] | 4 bytes, the access password for the tag. |
|  | Target | unsigned char | [in] | 1 byte, query the Target value of the tag. 0 – Target A； 1 – Target B。 |
|  | InAnt | unsigned char | [in] | 1 byte, the antenna number to be queried this time. 0x80- Antenna 1;0x81- Antenna 2; 0x82- Antenna 3;0x83- Antenna 4; 0x84- Antenna 5;0x85- Antenna 6; 0x86- Antenna 7;0x87- Antenna 8; 0x88- Antenna 9;0x89- Antenna 10; 0x8A - Antenna 11;0x8B - Antenna 12; Single port reader/writer fixed at 0x80. |
|  | Scantime | unsigned char | [in] | 1 byte, the maximum time for this command query, range 0-255 (* 100ms). When the time is 0, it means there is no limit. |
|  | FastFlag | unsigned char | [in] | 1 byte, a flag for quick query. 0- Not enabled, no need to specify Target, Inant, Scantime has 3 parameters, default is 0; 1- Enable, at this time, you need to specify three parameters: Target, Inant, and Scantime. |
|  | pEPCList | unsigned char* | [out] | The returned label data is in the following format: Rolling code+EPC/data length+EPC/data+RSSI.When there are multiple labels, they are returned by arranging several data in the same format in order.The highest bit of the rolling code is 0, indicating that it is currently EPC, and the highest bit is 1, indicating storage data. For data with the same label, upload the EPC number first, and then upload the storage data. If the lower 7 bits are adjacent, it can be confirmed that it is the same data. If it is not adjacent, it is not the same label data. The lower 7 bits of the rolling code range from 0 to 127, and the serial number is incremented every time it is uploaded. If it exceeds 127, it will start again from 0If the length of bit6 is 1, add 7 bytes of phase frequency information after the RSSI of the tag, starting phase (2 bytes)+ending phase (2 bytes)+frequency (3 bytes) |
|  | Ant | unsigned char* | [out] | 1 byte, query the antenna of the tag. 4-port reader/writer, each bit represents the antenna number, such as 0x04, binary is 0000 0100, indicating that antenna 3 queried the tag. 12 port reader/writer, corresponding to antenna 1 to antenna 12 according to the numbers 0-11. |
|  | Totallen | int* | [out] | The total length of data received by pEPLSist. |
|  | CardNum | int* | [out] | The number of tags found in this query. Note: When using callback method to return, Totallen and CardNum are both 0. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0x01: Command execution ends and returns the queried electronic tag data; 0x02: Inquiry time ends, command execution forces exit, and returns the queried label data 0xF8: Antenna connection detection error, the current antenna connection may have been disconnected; For the remaining values, please refer to the table of error codes returned. |  |  |  |
| Reference code | None. |  |  |  |

### 3.2.3.InitRFIDCallBack

| definition | void InitRFIDCallBack(pRFIDCallBack pUIDback,bool isBackUID,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | Initialize callback function. After initializing this API, tags obtained from query commands and mixed query commands can be uploaded through callback. |  |  |  |
| parameter | name | type | direction | notes |
|  | pUIDback | pRFIDCallBack | [in] | Callback interface pointer typedef struct _RFIDTag { unsigned char PacketParam;//Packet type flag unsigned char LEN;//Length char* UID;//EPC or UID int phase_begin; int phase_end; unsigned char RSSI; Int Freqkhz; unsigned char ANT; int FrmHandle; }MyStruct,*RFIDTagStruct; Typedefvoid(CALLBACK*pRFIDCallBack)( RFIDTagStruct lpRfid, INT32 nEvt); |
|  | isBackUID | bool | [in] | - 0-callback and interface array return. - Callback returns. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return | not have |  |  |  |
| Reference code |  |  |  |  |

### 3.2.4.ReadData_G2

| definition | int ReadData_G2(unsigned char * ComAddr, unsigned char *EPC, unsigned char ENum, unsigned char Mem, unsigned char WordPtr, unsigned char Num, unsigned char *Password, unsigned char MaskMem, unsigned char *MaskAdr, unsigned char MaskLen, unsigned char *MaskData, unsigned char *Data, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command reads data from the entire or partial reserved area, EPC memory, TID memory, or user memory of the tag.Read from the specified address in words. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | EPC | unsigned char* | [in] | ENum, the EPC number of the label. |
|  | ENum | unsigned char | [in] | 1 byte, the length of the EPC number word of the tag, when VNet=255, use masking method. |
|  | Mem | unsigned char | [in] | 1 byte, select the storage area to be read, 0x00- Reserved area; 0x01- EPC storage area; 0x02- TID storage area; 0x03- User Storage Area.Other values are retained.If other values appear in the command, a message indicating parameter error will be returned.. |
|  | WordPtr | unsigned char | [in] | 1 byte, the starting word address to be read. |
|  | Num | unsigned char | [in] | 1 byte, the length of the word to be read. |
|  | Password | unsigned char* | [in] | 4 bytes, the access password for the tag. |
|  | MaskMem | unsigned char | [in] | 1 byte, masked area. 1- EPC storage area; 2- TID storage area; 3- User storage area. |
|  | MaskAdr | unsigned char* | [in] | 2 bytes, the starting bit address of the mask (in bits).Range 0~16383. |
|  | MaskLen | unsigned char | [in] | 1 byte, the bit length of the mask (in bits). |
|  | MaskData | unsigned char* | [in] | N bytes, masked data.N is equal to MaskLen/8.If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded to the nearest integer plus 1.If it's not enough, add 0 at the low level. |
|  | Data | unsigned char* | [out] | Num words, read label data. |
|  | ErrorCode | int * | [out] | When the return value is 0xFC, the label error message is returned. Please refer to the Error Code table. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | None. |  |  |  |

### 3.2.5.WriteData_G2

| definition | int WriteData_G2(unsigned char *ComAdr, unsigned char * EPC, unsigned char Wnum, unsigned char Enum, unsigned char Mem, unsigned char WordPtr,unsigned char*Writedata,unsigned char* Password, unsigned char MaskMem,unsigned char*MaskAdr,unsigned char MaskLen,unsigned char * MaskData,int * errorcode,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command can write several words to reserved memory, EPC memory, TID memory, or user memory at once. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | EPC | unsigned char* | [in] | ENum, the EPC number of the label. |
|  | Wnum | unsigned char | [in] | 1 byte, the number of words to be written, using a mask when VNet=255. |
|  | Enum | unsigned char | [in] | 1 byte, the EPC number word length of the tag.When enum=255, use masking method. |
|  | Mem | unsigned char | [in] | 1 byte, select the storage area to be written, 0x00- Reserved area; 0x01- EPC storage area; 0x02- TID storage area; 0x03- User Storage Area.Other values are retained.If other values appear in the command, a message indicating parameter error will be returned.. |
|  | WordPtr | unsigned char | [in] | 1 byte, the starting word address to be written. |
|  | Writedata | unsigned char* | [in] | WNum bytes, the data to be written. |
|  | Password | unsigned char* | [in] | 4 bytes, the access password for the tag. |
|  | MaskMem | unsigned char | [in] | 1 byte, masked area. 1- EPC storage area; 2- TID storage area; 3- User storage area. |
|  | MaskAdr | unsigned char* | [in] | 2 bytes, the starting bit address of the mask (in bits).Range 0~16383. |
|  | MaskLen | unsigned char | [in] | 1 byte, the bit length of the mask (in bits). |
|  | MaskData | unsigned char* | [in] | N bytes, masked data.N is equal to MaskLen/8.If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded to the nearest integer plus 1.If it's not enough, add 0 at the low level. |
|  | ErrorCode | int * | [out] | When the return value is 0xFC, the label error message is returned. Please refer to the Error Code table. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | None. |  |  |  |

### 3.2.6.BlockWrite_G2

| definition | int BlockWrite_G2(unsigned char *ComAdr, unsigned char * EPC, unsigned char Wnum, unsigned char Enum, unsigned char Mem, unsigned char WordPtr,unsigned char*Writedata,unsigned char* Password, unsigned char MaskMem,unsigned char*MaskAdr,unsigned char MaskLen,unsigned char * MaskData,int * errorcode,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command can write several words to reserved memory, EPC memory, TID memory, or user memory at once. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | EPC | unsigned char* | [in] | ENum, the EPC number of the label. |
|  | Wnum | unsigned char | [in] | 1 byte, the number of words to be written. |
|  | Enum | unsigned char | [in] | 1 byte, the EPC number word length of the tag.When enum=255, use masking method. |
|  | Mem | unsigned char | [in] | 1 byte, select the storage area to be written, 0x00- Reserved area; 0x01- EPC storage area; 0x02- TID storage area; 0x03- User Storage Area.Other values are retained.If other values appear in the command, a message indicating parameter error will be returned.. |
|  | WordPtr | unsigned char | [in] | 1 byte, the starting word address to be written. |
|  | Writedata | unsigned char* | [in] | WNum bytes, the data to be written. |
|  | Password | unsigned char* | [in] | 4 bytes, the access password for the tag. |
|  | MaskMem | unsigned char | [in] | 1 byte, masked area. 1- EPC storage area; 2- TID storage area; 3- User storage area. |
|  | MaskAdr | unsigned char* | [in] | 2 bytes, the starting bit address of the mask (in bits).Range 0~16383. |
|  | MaskLen | unsigned char | [in] | 1 byte, the bit length of the mask (in bits). |
|  | MaskData | unsigned char* | [in] | N bytes, masked data.N is equal to MaskLen/8.If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded to the nearest integer plus 1.If it's not enough, add 0 at the low level. |
|  | ErrorCode | int * | [out] | When the return value is 0xFC, the label error message is returned. Please refer to the Error Code table. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | None. |  |  |  |

### 3.2.7.ExtReadData_G2

| definition | int ExtReadData_G2(unsigned char * ComAddr, unsigned char *EPC, unsigned char ENum, unsigned char Mem, unsigned char* WordPtr, unsigned char Num, unsigned char *Password, unsigned char MaskMem, unsigned char *MaskAdr, unsigned char MaskLen, unsigned char *MaskData, unsigned char *Data, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command reads data from the entire or partial reserved area, EPC memory, TID memory, or user memory of the tag.Read from the specified address in words. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | EPC | unsigned char* | [in] | ENum, the EPC number of the label. |
|  | ENum | unsigned char | [in] | 1 byte, the length of the EPC number word of the tag, when VNet=255, use masking method. |
|  | Mem | unsigned char | [in] | 1 byte, select the storage area to be read, 0x00- Reserved area; 0x01- EPC storage area; 0x02- TID storage area; 0x03- User Storage Area.Other values are retained.If other values appear in the command, a message indicating parameter error will be returned.. |
|  | WordPtr | unsigned char* | [in] | 2 bytes, the starting word address to be read, with the higher byte coming first. |
|  | Num | unsigned char | [in] | 1 byte, the length of the word to be read. |
|  | Password | unsigned char* | [in] | 4 bytes, the access password for the tag. |
|  | MaskMem | unsigned char | [in] | 1 byte, masked area. 1- EPC storage area; 2- TID storage area; 3- User storage area. |
|  | MaskAdr | unsigned char* | [in] | 2 bytes, the starting bit address of the mask (in bits).Range 0~16383. |
|  | MaskLen | unsigned char | [in] | 1 byte, the bit length of the mask (in bits). |
|  | MaskData | unsigned char* | [in] | N bytes, masked data.N is equal to MaskLen/8.If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded to the nearest integer plus 1.If it's not enough, add 0 at the low level. |
|  | Data | unsigned char* | [out] | Num words, read label data. |
|  | ErrorCode | int * | [out] | When the return value is 0xFC, the label error message is returned. Please refer to the Error Code table. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | None. |  |  |  |

### 3.2.8.ExtWriteData_G2

| definition | int WriteData_G2(unsigned char *ComAdr, unsigned char * EPC, unsigned char Wnum, unsigned char Enum, unsigned char Mem, unsigned char* WordPtr,unsigned char*Writedata,unsigned char* Password, unsigned char MaskMem,unsigned char*MaskAdr, unsigned char MaskLen,unsigned char * MaskData,int * errorcode,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command can write several words to reserved memory, EPC memory, TID memory, or user memory at once. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | EPC | unsigned char* | [in] | ENum, the EPC number of the label. |
|  | Wnum | unsigned char | [in] | 1 byte, the number of words to be written. |
|  | Enum | unsigned char | [in] | 1 byte, the EPC number word length of the tag.When enum=255, use masking method. |
|  | Mem | unsigned char | [in] | 1 byte, select the storage area to be written, 0x00- Reserved area; 0x01- EPC storage area; 0x02- TID storage area; 0x03- User Storage Area.Other values are retained.If other values appear in the command, a message indicating parameter error will be returned.. |
|  | WordPtr | unsigned char* | [in] | 2 bytes, the starting word address to be written. |
|  | Writedata | unsigned char* | [in] | WNum bytes, the data to be written. |
|  | Password | unsigned char* | [in] | 4 bytes, the access password for the tag. |
|  | MaskMem | unsigned char | [in] | 1 byte, masked area. 1- EPC storage area; 2- TID storage area; 3- User storage area. |
|  | MaskAdr | unsigned char* | [in] | 2 bytes, the starting bit address of the mask (in bits).Range 0~16383. |
|  | MaskLen | unsigned char | [in] | 1 byte, the bit length of the mask (in bits). |
|  | MaskData | unsigned char* | [in] | N bytes, masked data.N is equal to MaskLen/8.If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded to the nearest integer plus 1.If it's not enough, add 0 at the low level. |
|  | ErrorCode | int * | [out] | When the return value is 0xFC, the label error message is returned. Please refer to the Error Code table. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | None. |  |  |  |

### 3.2.9.WriteEPC_G2

| definition | int WriteEPC_G2(unsigned char * ComAddr,unsigned char *Password, unsigned char *EPC, unsigned char ENum, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command writes the EPC number to the electronic tag.When writing, there can only be one electronic tag within the effective range of the antenna. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | Password | unsigned char* | [in] | 4 bytes, the access password for the tag. |
|  | EPC | unsigned char* | [in] | Enum bytes, the EPC number to be written. |
|  | ENum | unsigned char | [in] | 1 byte, the length of the label word written. |
|  | ErrorCode | int * | [out] | When the return value is 0xFC, the label error message is returned. Please refer to the Error Code table. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | None. |  |  |  |

### 3.2.10.BlockErase_G2

| definition | int BlockErase_G2(unsigned char * ComAddr, unsigned char *EPC, unsigned char ENum, unsigned char Mem, unsigned char WordPtr, unsigned char Num, unsigned char *Password, unsigned char MaskMem, unsigned char *MaskAdr, unsigned char MaskLen, unsigned char *MaskData, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command can erase several words from the reserved memory, EPC memory, TID memory, or user memory of the tag. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | EPC | unsigned char* | [in] | ENum, the EPC number of the label. |
|  | ENum | unsigned char | [in] | 1 byte, the length of the EPC number word of the tag, when VNet=255, use masking method. |
|  | Mem | unsigned char | [in] | 1 byte, select the storage area to be erased, 0x00- Reserved area; 0x01- EPC storage area; 0x02- TID storage area; 0x03- User Storage Area.Other values are retained.If other values appear in the command, a message indicating parameter error will be returned.. |
|  | WordPtr | unsigned char* | [in] | 1 byte, the starting word address to be erased. |
|  | Num | unsigned char | [in] | 1 byte, the length of the word to be erased. |
|  | Password | unsigned char* | [in] | 4 bytes, the access password for the tag. |
|  | MaskMem | unsigned char | [in] | 1 byte, masked area. 1- EPC storage area; 2- TID storage area; 3- User storage area. |
|  | MaskAdr | unsigned char* | [in] | 2 bytes, the starting bit address of the mask (in bits).Range 0~16383. |
|  | MaskLen | unsigned char | [in] | 1 byte, the bit length of the mask (in bits). |
|  | MaskData | unsigned char* | [in] | N bytes, masked data.N is equal to MaskLen/8.If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded to the nearest integer plus 1.If it's not enough, add 0 at the low level. |
|  | ErrorCode | int * | [out] | When the return value is 0xFC, the label error message is returned. Please refer to the Error Code table. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | None. |  |  |  |

### 3.2.11.Lock_C2

| definition | int Lock_G2(unsigned char * ComAddr,unsigned char *EPC, unsigned char ENum, unsigned char select, unsigned char setprotect, unsigned char *Password,unsigned char MaskMem,unsigned char *MaskAdr,unsigned char MaskLen,unsigned char *MaskData, int *Errorcode,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command can set the reserved area as read-write, always read-write, password read-write, and always read-write. It can respectively set EPC memory, TID memory, and user memory as writable, always writable, password writable, and never writable.EPC memory, TID memory, or user memory are always readable.Moreover, TID memory is read-only and can never be written to. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | EPC | unsigned char* | [in] | ENum, the EPC number of the label. |
|  | ENum | unsigned char | [in] | 1 byte, the length of the EPC number word of the tag, when VNet=255, use masking method. |
|  | select | unsigned char | [in] | 1 byte, 0x00- Control Kill password read-write protection settings. 0x01- Control access password read-write protection settings. 0x02- Control EPC memory read-write protection settings. 0x03- Control TID memory read-write protection settings. 0x04- Control user memory read/write protection settings. Other values are reserved, and if the reader/writer receives other values, it will return a message indicating a parameter error. |
|  | setprotect | unsigned char | [in] | 1 byte, When Select is 0x00 or 0x01, the meaning represented by the Set Protect value is as follows: 0x00- Set to read-write 0x01- Set to read/write permanently 0x02- Set to read and write with password 0x03- Set to never read/write When Select is 0x02, 0x03, 0x04, the meaning represented by the Set Protect value is as follows: 0x00- Set as writable 0x01- Set to always writable 0x02- Set as password writable 0x03- Set to never be writable. Note: When select and setprotect are both 0xFF, lock the U9 type label |
|  | Password | unsigned char* | [in] | 4 bytes, the access password for the tag. |
|  | MaskMem | unsigned char | [in] | 1 byte, masked area. 1- EPC storage area; 2- TID storage area; 3- User storage area. |
|  | MaskAdr | unsigned char* | [in] | 2 bytes, the starting bit address of the mask (in bits).Range 0~16383. |
|  | MaskLen | unsigned char | [in] | 1 byte, the bit length of the mask (in bits). |
|  | MaskData | unsigned char* | [in] | N bytes, masked data.N is equal to MaskLen/8.If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded to the nearest integer plus 1.If it's not enough, add 0 at the low level. |
|  | ErrorCode | int * | [out] | When the return value is 0xFC, the label error message is returned. Please refer to the Error Code table. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | None. |  |  |  |

### 3.2.12.KillTag_G2

| definition | int KillTag_G2(unsigned char * ComAddr, unsigned char *EPC, unsigned char ENum, unsigned char *Killpwd, unsigned char MaskMem, unsigned char *MaskAdr, unsigned char MaskLen, unsigned char *MaskData, int *Errorcode, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to destroy tags.After the tag is destroyed, the reader's commands will never be processed again. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | EPC | unsigned char* | [in] | ENum, the EPC number of the label. |
|  | ENum | unsigned char | [in] | 1 byte, the length of the EPC number word of the tag, when VNet=255, use masking method. |
|  | Killpwd | unsigned char* | [in] | 4 bytes, the kill password for the tag. |
|  | MaskMem | unsigned char | [in] | 1 byte, masked area. 1- EPC storage area; 2- TID storage area; 3- User storage area. |
|  | MaskAdr | unsigned char* | [in] | 2 bytes, the starting bit address of the mask (in bits).Range 0~16383. |
|  | MaskLen | unsigned char | [in] | 1 byte, the bit length of the mask (in bits). |
|  | MaskData | unsigned char* | [in] | N bytes, masked data.N is equal to MaskLen/8.If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded to the nearest integer plus 1.If it's not enough, add 0 at the low level. |
|  | ErrorCode | int * | [out] | When the return value is 0xFC, the label error message is returned. Please refer to the Error Code table. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | None. |  |  |  |

### 3.2.13.SingleTagInventory_G2

| definition | int SingleTagInventory_G2(unsigned char * ComAddr, unsigned char* EPC, int *EPCLength,int *CardNum,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | Check if there are electronic tags within the valid range that comply with the protocol. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | EPC | unsigned char* | [out] | EPCLength+2 bytes, Return the EPC number of the tag in the format of antenna number+EPC number+RSSI |
|  | EPCLength | int* | [out] | 1 byte, the length of the EPC number returned. |
|  | CardNum | int* | [out] | 1 byte, the number of returned labels. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | None. |  |  |  |

### 3.2.14.StartRead

| definition | int StartRead(unsigned char * ComAddr, unsigned char Target, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | Start quick inventory, and the inventory data will be returned through the pRFIDCallBack callback. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | Target | unsigned char | [in] | Marked, 0-Target A, 1-Target B |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | None. |  |  |  |

### 3.2.15.StopRead

| definition | int StopRead(unsigned char * ComAddr, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | Stop rapid inventory. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | None. |  |  |  |

### 3.2.16.Authenticate_G2

| definition | int Authenticate_G2(unsigned char * ComAddr,unsigned char *EPC,unsigned char ENum,unsigned char sendRep,unsigned char increpLen,unsigned char CSI,unsigned char MsgLen,unsigned char *MsgData,unsigned char *Password,unsigned char MaskMem, unsigned char *MaskAdr,unsigned char MaskLen,unsigned char *MaskData,unsigned char *auth_rep_len,unsigned char *auth_rep_data,int *Errorcode,int FrmHandle);; |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used for anti-counterfeiting authentication of Impinj tags. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | EPC | unsigned char* | [in] | ENum, the EPC number of the label. |
|  | ENum | unsigned char | [in] | 1 byte, the length of the EPC number word of the tag, when VNet=255, use masking method. |
|  | sendRep | unsigned char | [in] | 1 byte, 0x00: Send and store the result;0x01: Send without storing results. |
|  | increpLen | unsigned char | [in] | 1 byte, 0x00: omit response length;0x01: Contains response length. |
|  | CSI | unsigned char | [in] | 1 byte, password suite. |
|  | MsgLen | unsigned char | [in] | The message length must be 48 bits |
|  | MsgData | unsigned char* | [in] | Message data, 6-digit head+42 bit random number |
|  | Password | unsigned char* | [in] | 4 bytes, the access password for the tag. |
|  | MaskMem | unsigned char | [in] | 1 byte, masked area. 1- EPC storage area; 2- TID storage area; 3- User storage area. |
|  | MaskAdr | unsigned char* | [in] | 2 bytes, the starting bit address of the mask (in bits).Range 0~16383. |
|  | MaskLen | unsigned char | [in] | 1 byte, the bit length of the mask (in bits). |
|  | MaskData | unsigned char* | [in] | N bytes, masked data.N is equal to MaskLen/8.If MaskLen is not an integer multiple of 8, then n is [MaskLen/8] rounded to the nearest integer plus 1.If it's not enough, add 0 at the low level. |
|  | auth_rep_len | unsigned char* | [out] | Return data length |
|  | auth_rep_data | unsigned char* | [out] | Return label authentication data. |
|  | ErrorCode | int * | [out] | When the return value is 0xFC, the label error message is returned. Please refer to the Error Code table. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | None. |  |  |  |

3.3.Reader/writer custom commands

### 3.3.1.GetReaderInformation

| definition | int GetReaderInformation(unsigned char* ComAdr,unsigned char* VersionInfo, unsigned char* ReaderType, unsigned char* TrType, unsigned char* dmaxfre, unsigned char* dminfre, unsigned char* powerdBm, unsigned char* ScanTime, unsigned char*Ant, unsigned char*BeepEn, unsigned char*OutputRep, unsigned char*CheckAnt, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | Obtain information about the reader/writer, including multiple pieces of information such as the reader/writer address (ComAdr) and the reader/writer software version (Versions Info) |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | 1 byte, using the reader/writer address returned by the connection reader/writer. |
|  | VersionInfo | unsigned char* | [out] | 2 bytes, reader version information.The first byte is the version number, and the second byte is the sub version number handle. |
|  | ReaderType | unsigned char* | [out] | 1 byte, reader/writer type code. |
|  | TrType | unsigned char* | [out] | 1 byte, the reader/writer supports the protocol. |
|  | dmaxfre | unsigned char* | [out] | 1 byte, Bit7-Bit6 is used for frequency band setting;Bit5-Bit0 represents the maximum operating frequency of the current reader/writer. |
|  | dminfre | unsigned char* | [out] | 1 byte, Bit7-Bit6 is used for frequency band setting;Bit5-Bit0 represents the minimum frequency point at which the current reader/writer is operating. |
|  | powerdBm | unsigned char* | [out] | 1 byte, output power of the reader/writer.The range is 0 to 30, in dBm units. |
|  | ScanTime | unsigned char* | [out] | 1 byte, query time.After receiving the inquiry command, the reader/writer will respond to the upper computer within the inquiry time. |
|  | Ant | unsigned char* | [out] | 1 byte, antenna configuration information. |
|  | BeepEn | unsigned char* | [out] | 1 byte, buzzer switch information. |
|  | OutputRep | unsigned char* | [out] | 1 byte, notifying the output port parameters. |
|  | CheckAnt | unsigned char* | [out] | 1 byte, antenna detection information. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

Note: The frequency band settings are shown in the following table:

| MaxFre(Bit7) | MaxFre(Bit6) | MinFre(Bit7) | MinFre(Bit6) | FreqBand |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 1 | 0 | Ukraine band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| 1 | 1 | 0 | 0 | US band3 |

### 3.7.2 SetAddress

| definition | int SetAddress(unsigned char * ComAddr, unsigned char ComAdrData, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | After executing this command, the reader/writer will change the reader/writer address to the value given by the user and write this value to the EEPROM for storage.The default value at the factory is 0x00.The allowed modification range for users is 0x00~0xFE.When the value written by the user is 0xFF, the reader/writer will automatically revert to the default value 0x00. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | ComAdrData | unsigned char | [in] | 1 byte, the new address to be modified. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.3 SetInventoryScanTime

| definition | int SetInventoryScanTime(unsigned char *address, unsigned char ScanTime, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to set the maximum response time for query commands. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | ScanTime | unsigned char | [in] | 1 byte, query command response time, range 0 to 255. When set to 0, the parameter is invalid and waits until the round is completed. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.4.SetRfPower

| definition | int SetRfPower(unsigned char *address, unsigned char PowerDbm, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | Set the power of the reader/writer. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | PowerDbm | unsigned char | [in] | 1 byte, reader/writer power, range 0~33dBm.The highest bit Bit7 is set to 0, and it is set to save in case of power failure. If it is set to 1, it will not be saved in case of power failure. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.5.SetRegion

| definition | int SetRegion(unsigned char *address, unsigned char dmaxfre, unsigned char dminfre, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command sets the upper and lower limits of the frequency at which the reader/writer operates.The upper limit frequency must be greater than or equal to the lower limit frequency. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | dmaxfre | unsigned char | [in] | 1 byte, Bit7-Bit6 is used for frequency band setting;Bit5-Bit0 represents the maximum operating frequency of the current reader/writer. |
|  | dminfre | unsigned char | [in] | 1 byte, Bit7-Bit6 is used for frequency band setting;Bit5-Bit0 represents the minimum frequency point at which the current reader/writer is operating. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

Note: The frequency band settings are shown in the following table:

| MaxFre(Bit7) | MaxFre(Bit6) | MinFre(Bit7) | MinFre(Bit6) | FreqBand |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 1 | 0 | Ukraine band |
| 1 | 0 | 0 | 0 | Chinese band1 |
| 1 | 1 | 0 | 0 | US band3 |

Calculation formulas for each frequency band:

Chinese band 2: 		Fs=920.125+N * 0.25 (MHz), where N ∈ [0,19].

US band: 			Fs=902.75+N * 0.5 (MHz), where N ∈ [0,49].

Korean band: 			Fs=917.1+N * 0.2 (MHz), where N ∈ [0,31].

EU band: Fs=865.1+N * 0.2 (MHz), 				where N ∈ [0, 14].

Ukraine band: Fs=868.0+N * 0.1 (MHz), 			where N ∈ [0,6].

Chinese band 1: 		Fs=840.125+N * 0.25 (MHz), where N ∈ [0,19].

US band3: 			Fs=902+N * 0.5 (MHz) where N ∈ [0,52].

### 3.7.6.SetBaudRate

| definition | int SetBaudRate(unsigned char * ComAddr, unsigned char baud, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | Set the communication baud rate of the reader/writer |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | baud | unsigned char | [in] | 1 byte, baud rate. 0 – 9600bps; 1 – 19200bps; 2 – 38400bps; 5 – 57600bps; 6 – 115200bps; 7 – 921600bps. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.7.BuzzerAndLEDControl

| definition | int BuzzerAndLEDControl(unsigned char * ComAddr, unsigned char AvtiveTime, unsigned char SilentTime, unsigned char Times, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to control the LED lights and buzzer to flash and beep in a certain pattern. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | AvtiveTime | unsigned char | [in] | 1 byte, LED light on and buzzer beep time (ActiveT * 50ms), default value is zero.0<=ActiveT<=255。 |
|  | SilentTime | unsigned char | [in] | 1 byte, LED light and buzzer silence time (SilentT * 50ms), default value is zero.0<= SilentT <=255。. |
|  | Times | unsigned char | [in] | 1 byte, the default value for the number of times the LED light is on and the buzzer beeps (0<=Times<=255) is zero. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.8.SetAntennaMultiplexing

| definition | int SetAntennaMultiplexing(unsigned char *ComAdr, unsigned char Ant, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used for configuring the antenna of a 4-port reader/writer. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | Ant | unsigned char | [in] | 1 byte, antenna configuration information.At least one antenna should be configured. Bit0: Antenna 1 configuration bit. Bit0=0, antenna 1 is turned off;Bit0=1, antenna 1 is turned on. Bit1: Antenna 2 configuration bit. Bit1=0, antenna 2 is turned off;Bit1=1, antenna 2 is turned on. Bit2: Antenna 3 configuration bit. Bit2=0, antenna 3 is turned off;Bit2=1, antenna 3 is turned on. Bit3: Antenna 4 configuration bit. Bit3=0, antenna 4 is turned off;Bit3=1, antenna 4 is turned on. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.9.SetAntenna

| definition | int SetAntenna(unsigned char *ComAdr, unsigned char SetOnce, unsigned char AntCfg1, unsigned char AntCfg2, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used for configuring the antenna of a 12 port reader/writer. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | SetOnce | unsigned char | [in] | 1 byte, indicating whether the current configuration is saved after power failure. 0x00- Power off save; 0x01- Not saved in case of power failure. |
|  | AntCfg1 | unsigned char | [in] | 1 byte, antenna configuration. Bit0: Antenna 9 configuration bit. Bit0=0, antenna 9 is turned off;Bit0=1, antenna 9 is turned on. Bit1: Antenna 10 configuration bit. Bit1=0, antenna 10 is turned off;Bit1=1, antenna 10 is turned on. Bit2: Antenna 11 configuration bit. Bit2=0, antenna 11 is turned off;Bit2=1, antenna 11 is turned on. Bit3: Antenna 12 configuration bit. Bit3=0, antenna 12 is turned off;Bit3=1, antenna 12 is turned on. Bit4: Antenna 13 configuration bit. Bit4=0, antenna 13 is turned off;Bit4=1, antenna 13 is turned on. Bit5: Antenna 14 configuration bit. Bit5=0, antenna 14 is turned off;Bit5=1, antenna 14 is turned on. Bit6: Antenna 15 configuration bit. Bit6=0, antenna 15 is turned off;Bit6=1, antenna 15 is turned on. Bit7: Antenna 16 configuration bit. Bit7=0, antenna 16 is turned off;Bit7=1, antenna 16 is turned on. |
|  | AntCfg2 | unsigned char | [in] | 1 byte, antenna configuration. Bit0: Antenna 1 configuration bit. Bit0=0, antenna 1 is turned off;Bit0=1, antenna 1 is turned on. Bit1: Antenna 2 configuration bit. Bit1=0, antenna 2 is turned off;Bit1=1, antenna 2 is turned on. Bit2: Antenna 3 configuration bit. Bit2=0, antenna 3 is turned off;Bit2=1, antenna 3 is turned on. Bit3: Antenna 4 configuration bit. Bit3=0, antenna 4 is turned off;Bit3=1, antenna 4 is turned on. Bit4: Antenna 5 configuration bit. Bit4=0, antenna 5 is turned off;Bit4=1, antenna 5 is turned on. Bit5: Antenna 6 configuration bit. Bit5=0, antenna 6 is turned off;Bit5=1, antenna 6 is turned on. Bit6: Antenna 7 configuration bit. Bit6=0, antenna 7 is turned off;Bit6=1, antenna 7 is turned on. Bit7: Antenna 8 configuration bit. Bit7=0, antenna 8 is turned off;Bit7=1, antenna 8 is turned on. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.10.SetBeepNotification

| definition | int SetBeepNotification(unsigned char *ComAdr,unsigned char BeepEn, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to set the buzzer switch. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | BeepEn | unsigned char | [in] | 1 byte, 0x00- Buzzer off; 0x01- Buzzer on. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.11.SetReal_timeClock

| definition | int SetReal_timeClock(unsigned char *ComAdr, unsigned char *paramer, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to set the real-time clock of the reader/writer. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | paramer | unsigned char* | [in] | 6 bytes, namely year, month, day, hour, minute, and second. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.12.GetTime

| definition | int GetTime(unsigned char *ComAdr, unsigned char *paramer, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to read the real-time clock of the reader/writer. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | paramer | unsigned char* | [out | 6 bytes, namely year, month, day, hour, minute, and second. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.13.SetRelay

| definition | int SetRelay(unsigned char *ComAdr, unsigned char RelayTime, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to control the relay to close. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | RelayTime | unsigned char | [in] | 1 byte, relay closing time (1-255) * 50ms. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.14.GetGPIOStatus

| definition | int GetGPIOStatus(unsigned char *ComAdr, unsigned char *OutputPin, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to read the output of a general-purpose GPIO port, and the default output is high level. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | OutputPin | unsigned char* | [out] | One byte, Bit0-Bit3 represents the status of pins IN1-IN4, Bit4-Bit7 represents the status of Out1-Out4, and other bits are reserved. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.15.SetGPIO

| definition | int SetGPIO(unsigned char *ComAdr, unsigned char OutputPin, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to set the output status of GPIO port. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | OutputPin | unsigned char | [in] | 1 byte, GPIO ports (Out1-Out4 pins) output status.Bit0-Bit3 control the Out1-Out4 pins respectively, while Bit4-Bit7 are reserved. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.16.SetNotificationPulseOutput

| definition | int SetNotificationPulseOutput(unsigned char *ComAdr, unsigned char OutputRep, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to set whether to output a 2ms low pulse on the corresponding port when a label is queried. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | OutputRep | unsigned char | [in] | 1 byte, Bit0: corresponds to antenna 1 and Out1 pins.When Bit0=1, when antenna 1 queries the tag, the Out1 pin outputs a 2ms low pulse;When Bit0=0, there is no signal output. Bit1: Corresponding to antenna 2 and Out2 pins.When Bit1=1, when antenna 2 queries the tag, the Out2 pin outputs a 2ms low pulse;When Bit1=0, there is no signal output. Bit2: Corresponding to antenna 3 and Out3 pins.When Bit2=1, when antenna 3 queries the tag, the Out3 pin outputs a 2ms low pulse;When Bit2=0, there is no signal output. Bit3: Corresponding to antenna 4 and Out4 pins.When Bit3=1, when antenna 4 queries the tag, the Out4 pin outputs a 2ms low pulse;When Bit3=0, there is no signal output. Other bits are reserved, with a default value of 0. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.17.GetSeriaNo

| definition | int GetSeriaNo(unsigned char * ComAddr, unsigned char* SeriaNo, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to obtain the unique serial number of the 4-byte reader/writer. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | SeriaNo | unsigned char* | [out] | 4 bytes, reader serial number. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.18.SetCheckAnt

| definition | int SetCheckAnt(unsigned char *address, unsigned char CheckAnt, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to set whether antenna detection is performed during the process of reading and writing tags. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | CheckAnt | unsigned char | [in] | 1 byte, antenna detection switch. 0x00- Disable antenna detection during tag reading and writing process. 0x01- Enable antenna detection during tag reading and writing process. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.19.GetTagBufferInfo

| definition | int GetTagBufferInfo(unsigned char *ComAdr, unsigned char *Data, int *dataLength, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to retrieve all label information from the storage block of the reader/writer in active mode. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | Data | unsigned char* | [out] | DataLength bytes, the reader/writer stores all label information in the block. |
|  | dataLength | int* | [out] | The byte length of Data. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.20.GetModuleVersion

| definition | int GetModuleVersion(unsigned char * ComAddr, unsigned char *Version, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to obtain the version number of an internal module. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | Version | unsigned char* | [out] | 2 bytes, R2000 engine version number. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.21.SetFlashRom

| definition | int SetFlashRom(unsigned char * ComAddr, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to switch the reader/writer to R2000 engine upgrade mode. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.22.WriteRfPower

| definition | int WriteRfPower(unsigned char * ComAddr, unsigned char PowerDbm, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to set the power when writing labels. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | PowerDbm | unsigned char | [in] | 1 byte, write power of the reader/writer, 0-33;Bit7=0- not enabled; Bit7=1- Enable;. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.23.ReadRfPower

| definition | int ReadRfPower(unsigned char * ComAddr, unsigned char* PowerDbm, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to read the power when writing tags. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | PowerDbm | unsigned char* | [out] | 1 byte, write power of the reader/writer, 0-33;Bit7=0- not enabled; Bit7=1- Enable;. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.24.RetryTimes

| definition | int RetryTimes(unsigned char * ComAddr, unsigned char *Times, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to set the number of repetitions for data write failures. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | Times | unsigned char* | [out] | 1 byte, Bit7=1- Set the number of rewrites; Bit7=0- Read rewrite times;Range 0-7. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.25.SetReadMode

| definition | int SetReadMode(unsigned char *ComAdr,unsigned char ReadMode,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to set the real-time query mode of the reader/writer. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | ReadMode | unsigned char | [in] | 1 byte, 0- Response mode; 1- Real time inquiry mode; 2- Trigger mode. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.26.SetDRM

| definition | int SetDRM(unsigned char * ComAddr, unsigned char DRM, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to set the DRM status. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | DRM | unsigned char | [in] | 1 byte, DRM status, 0- Close; 1- Open. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.27.GetDRM

| definition | int GetDRM(unsigned char * ComAddr, unsigned char* DRM, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to read the DRM status. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | DRM | unsigned char* | [out] | 1 byte, DRM status, 0- Close; 1- Open. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.28.GetReaderTemperature

| definition | int GetReaderTemperature (unsigned char * ComAddr, unsigned char* PlusMinus,unsigned char* Temperature, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to read the current temperature of the reader/writer. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | PlusMinus | unsigned char* | [out] | 1 byte, temperature status, 0- minus zero; 1- Above zero. |
|  | Temperature | unsigned char* | [out] | 1 byte, temperature value, unit: ° C |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.29.MeasureReturnLoss

| definition | int MeasureReturnLoss(unsigned char * ComAddr, unsigned char* TestFreq, unsigned char Ant, unsigned char* ReturnLoss,int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to measure the return loss of the specified port of the reader/writer. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | TestFreq | unsigned char* | [in] | 4 bytes, measure frequency point. |
|  | Ant | unsigned char | [in] | 1 byte, measuring antenna port, 0- Antenna 1; 1- Antenna 2. 2- Antenna 3; 3- Antenna 4. |
|  | ReturnLoss | unsigned char* | [out] | 1 byte, return loss, in dB. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.30.SetRfPower

| definition | int SetRfPower(unsigned char *address, unsigned char* PowerDbm, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | Set the individual power for each antenna of the reader/writer. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | PowerDbm* | unsigned char* | [in] | 4 bytes, reader power, unsigned char0-unsigned char3 represent the power of antenna 1 to antenna 4, respectively.The antenna power parameters to be set for each byte from Bit0 to Bit6 range from 0 to 30, with a value of 30 resulting in an output power of approximately 1W. Bit7: Indicates whether the current configuration has been saved after power failure.Bit7=0 power failure save;Bit7=1 does not save in case of power failure. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.31.GetRfPower

| definition | int GetRfPower(unsigned char *address, unsigned char* PowerDbm, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | Obtain the individual power of each antenna of the reader/writer. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char* | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | PowerDbm* | unsigned char* | [out] | 4 bytes, reader power, unsigned char0-unsigned char3 represent the power of antenna 1 to antenna 4, respectively.The range is from 0 to 30, and when the value is 30, it is about 1W of output power. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.32.SetCfgParameter

| definition | int SetCfgParameter(unsigned char * ComAddr, unsigned char opt, unsigned char cfgNum, unsigned char * data,int len, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to configure extension parameters, limited to the Ex10 series only. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char * | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | opt | unsigned char | [in] | 0-Power off storage;1. Do not save in case of power failure; |
|  | cfgNum | unsigned char | [in] | The command data block is used to specify the parameter number to be set |
|  | data | unsigned char * | [in] | Set parameter content, specifically refer to the reader/writer parameter configuration table |
|  | len | int | [in] | Length of parameter content. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.33.GetCfgParameter

| definition | int GetCfgParameter (unsigned char *ComAddr, unsigned char cfgNum,unsigned char * data, int* len, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to read extended parameters, limited to the Ex10 series only. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char * | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | cfgNum | unsigned char | [in] | Used in the command data block to specify the parameter number to be read |
|  | data | unsigned char * | [out] | Read the parameter content, specifically refer to the reader/writer parameter configuration table |
|  | len | int * | [out] | Length of parameter content. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.34.SetProfile

| definition | int SetProfile (unsigned char *ComAddr, unsigned char* Profile, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to configure the profile, |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char * | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | Profile | unsigned char * | [in/out] | Set/read parameters. Bit7: Read/set profile flags. 0- Read profile number; 1- Set the profile number. Bit6: Power down save flag (Ex10 valid). 0- Save in case of power failure; 1- Do not save in case of power failure. Bit5~Bit0: profile number. If Bit7=0, Bit6~Bit0 will be ignored; If Bit7=1, Bit5 to Bit0 are the profile numbers to be set. The configuration of R2000 is as follows: Profle 0：Tari 25uS，FM0 40KHz； Profile 1: Tari 25uS, Miller 4 250KHz (recommended configuration, system default); Profile2：Tari 25uS，Miller 4 300KHz； Profile3：Tari 6.25uS，FM0 400KHz。 The configuration of Ex10 is as follows: Profile11: 640kHZ, FM0 ,Tari 7.5 us Profile1: 640kHZ, Miller2,Tari 7.5 us Profile15: 640kHZ, Miller4,Tari 7.5 us Profile12: 320kHZ, Miller2,Tari 15 us Profile3: 320kHZ, Miller2,Tari 20 us Profile5: 320kHZ, Miller4,Tari 20 us Profile7: 250kHZ, Miller4,Tari 20 us Profile13: 160kHZ, Miller8,Tari 20 us Profile50: 640kHZ, FM0 ,Tari 6.25 us Profile51: 640kHZ, Miller2,Tari 6.25 us Profile52: 426kHZ, FM0 ,Tari 15 us Profile53: 640kHZ, Miller4,Tari 7.5 us |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.35.StopImmediately

| definition | int StopImmediately(unsigned char *ComAddr, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to immediately stop the currently ongoing query command |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char * | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

### 3.7.36.SetExtProfile

| definition | int SetExtProfile(unsigned char* ComAdr, unsigned char Opt, int* Profile, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to set or read the internal profile parameter configuration of the Ex10 reader/writer.(Ex10 firmware version number>2) |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char * | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | Opt | unsigned char | [in] | 1 byte, operation tag, 0-read;1. Permanent setting;2- Temporary settings; |
|  | Profile | int* | [in/out] | 2 bytes, reader profile number. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

Profile Number:

1:  PR_ASK, pie: 2.0, tari_us: 7.5, lf_khz: 640, M: 2,

3:  PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 320, M: 2,

5:  PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 320, M: 4,

7:  PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 250, M: 4,

11: PR_ASK, pie: 2.0, tari_us: 7.5, lf_khz: 640, M: 1,

12: PR_ASK, pie: 2.0, tari_us: 15.0,lf_khz: 320, M: 2,

13: PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 160, M: 8,

15: PR_ASK, pie: 2.0, tari_us: 7.5, lf_khz: 640, M: 4,

102:PR_ASK, pie: 2.0, tari_us: 7.5, lf_khz: 640, M: 1,

103:DSB,    pie: 1.5, tari_us: 6.25,lf_khz: 640, M: 1,

104:DSB,    pie: 1.5, tari_us: 6.25,lf_khz: 320, M: 1,

120:DSB,    pie: 1.5, tari_us: 6.25,lf_khz: 640, M: 2,

123:PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 320, M: 2,

124:PR_ASK, pie: 2.0, tari_us: 7.5, lf_khz: 640, M: 2,

125:PR_ASK, pie: 2.0, tari_us: 15.0,lf_khz: 320, M: 2,

126:PR_ASK, pie: 1.5, tari_us: 12.5,lf_khz: 320, M: 2,

141:PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 320, M: 4,

146:PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 250, M: 4,

147:PR_ASK, pie: 2.0, tari_us: 7.5, lf_khz: 640, M: 4,

148:PR_ASK, pie: 1.5, tari_us: 7.5, lf_khz: 640, M: 4,

185:PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 160, M: 8,

202:PR_ASK, pie: 2.0, tari_us: 15.0,lf_khz: 426, M: 1,

203:PR_ASK, pie: 1.5, tari_us: 12.5,lf_khz: 426, M: 1,

205:PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 50,  M: 1,

222:PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 320, M: 2,

223:PR_ASK, pie: 2.0, tari_us: 15.0,lf_khz: 320, M: 2,

224:PR_ASK, pie: 1.5, tari_us: 12.5,lf_khz: 320, M: 2,

225:PR_ASK, pie: 2.0, tari_us: 15.0,lf_khz: 426, M: 2,

226:PR_ASK, pie: 1.5, tari_us: 12.5,lf_khz: 426, M: 2,

241:PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 320, M: 4,

244:PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 250, M: 4,

285:PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 160, M: 8,

302:PR_ASK, pie: 2.0, tari_us: 7.5, lf_khz: 640, M: 1,

323:PR_ASK, pie: 2.0, tari_us: 7.5, lf_khz: 640, M: 2,

324:PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 320, M: 2,

325:PR_ASK, pie: 2.0, tari_us: 15.0,lf_khz: 320, M: 2,

326:PR_ASK, pie: 1.5, tari_us: 12.5,lf_khz: 320, M: 2,

342:PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 320, M: 4,

343:PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 250, M: 4,

344:PR_ASK, pie: 2.0, tari_us: 7.5 ,lf_khz: 640, M: 4,

345:PR_ASK, pie: 1.5, tari_us: 7.5 ,lf_khz: 640, M: 4,

382:PR_ASK, pie: 2.0, tari_us: 20.0,lf_khz: 160, M: 8,

4123:PR_ASK,pie: 2.0, tari_us: 20.0,lf_khz: 320, PSK2,

4124:PR_ASK,pie: 2.0, tari_us: 7.5, lf_khz: 640, BPSK2,

4141:PR_ASK,pie: 2.0, tari_us: 20.0,lf_khz: 320, BPSK4,

4146:PR_ASK,pie: 2.0, tari_us: 20.0,lf_khz: 250, BPSK4,

4148:PR_ASK,pie: 1.5, tari_us: 7.5, lf_khz: 640, BPSK4,

4185:PR_ASK,pie: 2.0, tari_us: 20.0,lf_khz: 160, BPSK8,

5123:PR_ASK,pie: 2.0, tari_us: 20.0,lf_khz: 320, M+B2,

5124:PR_ASK,pie: 2.0, tari_us: 7.5, lf_khz: 640, M+B2,

5141:PR_ASK,pie: 2.0, tari_us: 20.0,lf_khz: 320, M+B4,

5146:PR_ASK,pie: 2.0, tari_us: 20.0,lf_khz: 250, M+B4,

5148:PR_ASK,pie: 1.5, tari_us: 7.5, lf_khz: 640, M+B4,

5185:PR_ASK,pie: 2.0, tari_us: 20.0,lf_khz: 160, M+B8

### 3.7.37.GetEx10Version

| definition | int GetEx10Version(unsigned char* ComAdr, unsigned char* TVersion, unsigned char* Ex10Type, int FrmHandle); |  |  |  |
| --- | --- | --- | --- | --- |
| describe | This command is used to obtain the internal version number of Ex10. |  |  |  |
| parameter | name | type | direction | notes |
|  | ComAddr | unsigned char * | [in/out] | Use the reader/writer address returned by the connected reader/writer. |
|  | TVersion | unsigned char* | [out] | 2 bytes, Ex10 firmware version number |
|  | Ex10Type | unsigned char* | [out] | 1 byte, chip type. |
|  | FrmHandle | int | [in] | Connect the handle returned by the reader/writer. |
| return (int) | 0: indicates success.Not equal to 0: indicates failure, please refer to the return value error code table. |  |  |  |
| Reference code | not have |  |  |  |

- 4. Reader/writer parameter configuration

All configuration parameters of the reader/writer are listed here, making it convenient for users to set and read them uniformly.

The reading of parameters uses the command to obtain the reader/writer parameters, and the setting of parameters uses the command to configure the reader/writer.After modifying the configuration, except for the network configuration that needs to be restarted to take effect, all other configurations will take effect immediately.

4.1 Controller Configuration Parameter Table

Configuration Table:

| Serial Number | Parameter Name | Length (unsigned char) | describe |
| --- | --- | --- | --- |
| 7 | Inquire about parameters | 3 | Parameter information used during the inquiry process |
| 8 | TagFocus | 1 | TagFocus switch |
| 9 | QAndSession | 2 | Q and Session query command parameters |
| 10 | TIDparam | 2 | TID inquiry parameters |
| 11 | MaskParam | N | Mask parameter |
| 25 | Impinj_Scan_param | 10 | Impinj scan command parameter |
| 29 | Impinj_ScanID_param | 10 | Impinj scan ID command parameter |
| 31 | Custom Profile | 6 | Customize the profile parameters of the profile scene mode |

4.2 Reader and writer parameter function description

### 4.2.1 CFG7: Inquiry Parameters

This configuration is used to set the parameters used by the reader/writer during the query process.

| unsigned char | 0 | 1 | 2 |
| --- | --- | --- | --- |
| Contents | Interval | DwellTime | Count |
| Default | 3 | 0 | 5 |

Interval: The interval time between queries, ranging from 0 to 6, with a unit of 10ms;

DwellTime: inquiry time, range 2-255, unit 100ms; fixed at 200ms when tagfocus switch is turned on.

Count: The number of inquiries, valid only for S1, S2, and S3, ranging from 0 to 5;

### 4.2.2 CFG: TagFocus switch

This configuration is used to set the switch for TagFocus.

| unsigned char | 0 |
| --- | --- |
| Contents | TagFocus |
| Default | 0 |

TagFocus: 0-Close, 1-Open.

### 4.2.3 CFG9：Q and Session

This configuration is used to set the anti-collision parameters used by the reader/writer during the fast query process.

| Byte | 0 | 1 |
| --- | --- | --- |
| Contents | Q | Session |
| Default | 6 | 1 |

Q: Range 0~15.

Session：0:S0；1:S1；2:S2；3:S3；255:Auto;

254:S-Pro; 253:S-RPT; 128:Custom-Profile;

4.2.4 CFG 10: TID Inquiry Parameters

This configuration is used to set the TID parameter used by the reader/writer during the fast query process.

| unsigned char | 0 | 1 |
| --- | --- | --- |
| Contents | AdrTID | LenTID |
| Default | 6 | 1 |

AdrTID: Inquire about the starting address of the TID area.

LenTID: Inquire about the word count of data in the TID area.The value of LenTID ranges from 0 to 15.When the value is 0, it indicates EPC query.

4.2.5 CFG 11: Mask Parameters

This configuration is used to set the mask parameters used by the reader/writer during the fast query process.

| unsigned char | MaskMem | MaskAdr | MaskLen | MaskData |
| --- | --- | --- | --- | --- |
| Contents | 1 | 2 | 1 | MaskLen |
| Default | 1 | 0x0020 | 0 | 0x00 |

MaskMem: One byte, masked area.0x01: EPC storage area;0x02: TID storage area;0x03: User storage area.Other values are retained.If other values appear in the command, a message indicating parameter error will be returned.

MaskAdr: Two bytes, the starting bit address of the mask (in bits).Range 0~16383.

MaskLen: One byte, the bit length of the mask (in bits).When the length is 0, it means that the masking function is not enabled.

MaskData: Mask data.The byte length of MaskData data is MaskLen/8.If MaskLen is not an integer multiple of 8, then the byte length of MaskData data is [MaskLen/8] rounded to the nearest integer plus 1.If it's not enough, add 0 at the low level.

4.2.6 CFG25：Impinj Scan Parameters

This configuration is used to set the Impinj Scan parameters.

| Byte | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Contents | N | Code | CR | Protection | ID | Copyto |
| Default | 15 | 1 | 3 | 2 | 3 | 0 |

N,: Default 0x0F;

Code: 0-Rfu;1-Antipodal;2-CCOneHalf;3-CCThreeQuarters.

CR: 0-ID32;1-ID16;2-StoredCRC;3-RN16.

Protection: 0-NoProtection;1-Parity;2-CRC5;3-CRC5Plus.

ID: 0-NoAckResponse;1-TMNPlusTSN; 2-Part EPC;3-Full.

Copyto: 0-All tag;1-S=1 tags only.

4.2.7 CFG29：Impinj ScanID Parameters

This configuration is used to set the Impinj ScanID parameter.

| Byte | 0 | 1 | 2-4 |
| --- | --- | --- | --- |
| Contents | ScanIDEnable | Appsize | AppID |
| Default | 0 | 0 | 0 |

ScanIDEnable: 0-disabled;1- Enable

Appsize: 0-Rfu;1-24bits;2-16bits;3-8bits;.

AppID: Mask value.

RFU: Reserved bytes

4.2.8 CFG 31: Scene Mode Custom Profile

This configuration is used to set the profile order for custom Profile profiles in scene mode.

| Byte | 0-1 | 2-3 | 4-5 |
| --- | --- | --- | --- |
| Contents | Profile0(2bytes) | Profile1(2bytes) | Profile2(2bytes) |
| Default | 103 | 241 | 285 |

Appendix 1. Return Value Error Code Table

| error code | describe |
| --- | --- |
| 0x00 | No errors, indicating successful API call. |
| 0x01 | Return before the end of the inquiry time. |
| 0x02 | The specified query time has overflowed. |
| 0x05 | The access password is incorrect. |
| 0x09 | Password destruction error. |
| 0x0A | The password for destruction cannot be all 0s. |
| 0x0B | The electronic tag does not support this command. |
| 0x0C | The access password for this command cannot be 0. |
| 0x0D | The electronic tag has already been set with read protection and cannot be reset. |
| 0x0E | The electronic tag is not set with read protection and does not need to be unlocked. |
| 0x10 | Byte space is locked, write failed. |
| 0x11 | Cannot be locked. |
| 0x12 | Already locked, cannot be locked again. |
| 0x13 | Parameter save failed, but the set value is valid until the read-write module is powered off. |
| 0x14 | Unable to adjust. |
| 0x15 | Return before the end of the inquiry time. |
| 0x17 | After this message, there are more messages. |
| 0x18 | The storage space of the read-write module is full. |
| 0x19 | The command is not supported electronically or the access password cannot be 0 |
| 0xF8 | Antenna detection error. |
| 0xF9 | Command execution error. |
| 0xFA | There is an electronic tag, but communication is not smooth and it cannot be operated. |
| 0xFB | No electronic tags available for operation. |
| 0xFC | The electronic tag returned an error code. |
| 0xFD | Command length error. |
| 0xFE | Illegal command. |
| 0xFF | Parameter error. |
| 0x30 | Communication error. |
| 0x33 | Communication is busy and the device is executing other commands. |
| 0x35 | port opened |
| 0x37 | Invalid handle. |

Appendix 2. Error Code Table

| error code | describe |
| --- | --- |
| 0x00 | Other errors, capture all errors that are not covered by other code. |
| 0x03 | PC values with memory exceeding limit or unsupported, specified storage location does not exist or marked The signature does not support PC value. |
| 0x04 | Memory lock, storage location lock is permanently locked and cannot be written to. |
| 0x0B | Insufficient power supply, label power supply is insufficient, unable to perform storage and write operations. |
| 0x0F | Non specific error, tags do not support specific error codes. |